Description of file renaming script: purpose is to rename all Pro2 fit files to reflect the participant number and training session.

- For an individual participant:

  - Download the participant files from the PrO2fit website in order, starting with the first session after familiarization, and place them in a subfolder called “data”.

  - Open the “PRO2_Fit_file renaming” script and change the subject name in the script to the appropriate subject name. (Make sure that the subject name is in quotes, see below)

  - Run the script by clicking “Run” in the Spyder IDE (or whatever IDE you are using).

- How the script works:

  - Set Participant ID

    - The script starts with a participant_ID (e.g., "MRS_07").

    - This ID will be used to name the folder and prefix all filenames.

  - Create Folder Structure

    - Locates the “data” folder (must be in the same directory as the script).

    - Creates a new subfolder inside data using the participant_ID.

  - Find & Sort Files

    - Identifies all files in the “data” folder excluding subfolders.

    - Sorts them by modification time (oldest to newest), assuming earlier files were collected first.

  - Rename and Move Files

    - Each file is renamed to follow the format:
ParticipantID_XX.ext (e.g., MRS_07_01.csv, MRS_07_02.csv, etc.)

    - Files are then moved into the new participant folder.

  - Create a Rename Log

    - A rename_log.txt is saved inside the participant’s subfolder.

    - It tracks each file's original name and new name for reference.

Description of the Pro2fit analysis script processes:

Methods for quantifying SMIP: (note running this script took about 1 minute for 24 participants)

- SMIP is calculated for each breath by taking the integral (using the trapezoid rule) to find the area under the pressure x time curve. This yields results in cmH2O*s.

Methods for quantifying air flow from the PrO2 fit:

Methods for quantifying air flow from the PrO2 fit:

Orifice equation: f= Cd*A* 2* ΔPρ

- f= air flow (m3/s)

- Cd = Discharge coefficient for air through a round, small diameter orifice

- A = area of the orifice (i.e. (π * (0.002)2) /4)

- ΔP = difference in pressure across the orifice (measured PrO2 fit pressure)

- ρ = air density (see below)

Air density estimation: 1.089416 kg/m3

- Air pressure: 92399.25 Pa (771 ft above sea level, i.e. Bloomington elevation)

- Room temperature 21.11 º C

- Average humidity indoors: 40%

Discharge coefficient: 0.84 from figure 111

- Diameter is closest to 2 mm and the reported temperature was 22º C

Methods for quantifying instantaneous power in Watts:

- Instantaneous power in Watts (joules per second) = pressure (in pascals) * f

Methods for work of each breath in Joules:

- Integral (using trapezoid rule) of power (Joules/s) and time (s)

Methods for quantifying MIP:

- MIP is calculated for each breath as the single highest pressure value reached during that breath and is reported in cmH2O

Methods for quantifying inspiratory time (IT):

- Inspiratory time is calculated for each breath as the elapsed time from the beginning of the breath segment to the final time point in that breath.

Methods for quantifying peak instantaneous power:

- Peak instantaneous power is calculated for each breath as the highest power value observed anywhere within that breath.

Methods for quantifying FIT:

- FIT is calculated by first estimating inspired volume from the flow signal, then identifying the time required to reach 500 mL. The pressure-time integral up to that 500 mL point is used together with whole-breath SMIP and inspiratory time to calculate FIT. If 500 mL is not reached, FIT is left blank for that breath.

Methods for quantifying time above 30%, 50%, and 80% MIP thresholds:

- For each file, the peak pressure from the first valid breath is used as the reference pressure. The script then calculates how long the pressure trace in each breath remains above 30%, 50%, and 80% of that first-breath peak, and sums those durations across the file.

Methods for quantifying total intervention time:

Total intervention time is calculated by summing the duration of every valid breath within a participant file or across all files for a participant, depending on the summary level being reported.

Methods for quantifying weekly summary variables:

- Training week is defined by grouping every 4 sessions into one week (As specified in the file configuration section).

- Weekly Total_Work_J and weekly Total_SMIP are calculated as sums across all sessions in that week.

- Weekly Max_Total_Work_J and Max_Total_SMIP are calculated as the highest total session values observed within that week.

- Weekly breath-level maximum values are calculated as the single highest breath values observed within that week for SMIP, MIP, IT, Work, FIT, and peak instantaneous power.

Methods for quantifying weekly percent time above 80% threshold:

- Pct_time_above_80_per_week is calculated as the total time spent above 80% of the first-breath reference pressure during that week divided by the total intervention time for that week, multiplied by 100.

Methods for quantifying participant-level percent time above threshold variables:

- Pct_time_above_30_pct, Pct_time_above_50_pct, and Pct_time_above_80_pct are calculated as the participant-level total time above each threshold divided by the participant-level total intervention time, multiplied by 100.

Methods for assigning group membership:

- Group is assigned from a fixed study ID lookup table built into the script and is carried into the weekly summary, SPSS-format summary, and long-format summary outputs.

References:

1.	Kayser JC, Shambaugh RL. Discharge coefficients for compressible flow through small-diameter orifices and convergent nozzles. Chem Eng Sci. 1991;46(7):1697-1711. doi:10.1016/0009-2509(91)87017-7